Step to use the AMR

Table of Contents

Step 1 connect to raspberry pi of amr	1

Step2 Configurations	2

TS	2

TOP	3

Move base	4

Step 3 RVIZ	6

# Step 1 connect to raspberry pi of amr

Open terminal with cnTL+ALT+T

Enter bx2 to ssh into the raspberry pi of the amr

# Step2 Configurations

Open another terminal and bx2 again

## TS

after that enter the command ts to timesync

## TOP

Enter the command top to see the processes

Kill 2 processes ur12_lidar and move_basic

Enter the command kill -SIGTERM 968

And kill -SIGTERM 946

The number depends on the PID and it changes everytime the system turns on and off

## Move base

Enter the command mb to run move base

This will mean that move base is properly running.

# Step 3 RVIZ

Enter the command rz2 in another terminal that has not entered the raspberry pi. In the local com vm

Rviz will start to load

Use the green arrow  to localize the amr to allow it to know where it is

The purple arrow is to make the amr move to where you want it to